TECHNICAL REPORT

Gyro-Controlled Swarm Bot

ESP32 / MicroPython Platform for Phone-Gyroscope Teleoperated Robotics

Prepared from: Gyro Bot Setup Guide & gyro_main.py firmware

Document date: 26 July 2026

Revision 1.0

# 1. Executive Summary

The Gyro-Controlled Swarm Bot is a two-wheel differential-drive robot built around an ESP32 microcontroller running MicroPython. The robot receives real-time orientation data (pitch and roll, encoded as a quaternion) from a smartphone's gyroscope over a self-hosted Wi-Fi access point, using the OSC (Open Sound Control) protocol carried over UDP. The ESP32 converts the two relevant quaternion components into throttle and steering commands, mixes them into independent left/right motor speeds, and drives the motors through a TB6612FNG dual H-bridge driver. An HC-SR04 ultrasonic sensor provides autonomous obstacle avoidance by cutting motor power whenever an object is detected within a configurable safe distance, and a command-timeout watchdog stops the robot if the phone's data stream is lost.

This report documents the complete system: the bill of materials, hardware wiring, firmware installation procedure, the structure and logic of the control firmware, the wireless communication protocol, the safety systems, the tunable control parameters, and the commissioning and troubleshooting procedures. The full firmware source (gyro_main.py) is reproduced in Appendix A.

# 2. System Overview

At a high level, the system is a closed control loop that runs entirely on the ESP32 at a fixed cadence of 40 ms (25 Hz), set by LOOP_PERIOD_MS. Each cycle, the firmware:

- Drains up to 8 pending UDP packets from the socket buffer and attempts to parse each as an OSC message.

- Extracts the quaternion's x and y components whenever a valid /quaternion message is found, and timestamps the update.

- Triggers an HC-SR04 ultrasonic ranging pulse and measures the echo return time to estimate distance to the nearest obstacle.

- Decides whether to drive or stop, based on obstacle distance and how long it has been since the last valid command.

- If clear to drive, converts the quaternion components into throttle and steering values, mixes them into left/right wheel speeds, and writes those speeds to the TB6612FNG via PWM and direction pins.

- Sleeps for whatever time remains in the 40 ms period, so the loop runs at a steady rate regardless of how long parsing and sensing took.

The system architecture is summarised in the block diagram below.

### 2.1 Signal & Data Flow

| Stage | Component | Function |
| --- | --- | --- |
| 1. Sensing (remote) | Phone gyroscope + Zig Sim app | Measures device orientation; encodes as OSC /quaternion (x, y, z, w) floats |
| 2. Transport | Wi-Fi (ESP32 SoftAP) + UDP | Carries OSC packets from phone to ESP32 on port 5005 |
| 3. Parsing | parse_osc_quaternion() | Decodes raw/bundled OSC packets into (x, y, z, w) floats |
| 4. Control law | component_to_control(), dead-zone + gain | Converts x → steering, y → throttle, with dead-zone and speed cap |
| 5. Mixing | main() loop | left = throttle + steering; right = throttle − steering |
| 6. Actuation | TB6612FNG + DC motors | PWM duty cycle sets speed; direction pins set forward/reverse |
| 7. Safety sensing | HC-SR04 ultrasonic | Measures distance to nearest obstacle each cycle |
| 8. Safety logic | obstacle_present / command_timed_out | Overrides drive command with stop_motors() when unsafe |

# 3. Bill of Materials

| Qty | Item | Notes |
| --- | --- | --- |
| 1 | ESP32 dev board | Any standard 30/38-pin board with CP2102 or CH340 USB-to-serial chip |
| 1 | TB6612FNG motor driver module | Dual H-bridge; drives both DC motors |
| 2 | DC gear motors + wheels | Voltage rating must match the battery pack |
| 1 | HC-SR04 ultrasonic sensor | Obstacle detection, 15 cm default threshold |
| 1 | Robot chassis | Two-wheel differential-drive layout |
| 1 | Battery pack | 2× 18650 or similar; voltage must match motor rating |
| — | Jumper wires | Male-male and male-female, as needed for wiring |
| 1 | USB cable (data-capable) | Required for flashing and initial testing — charge-only cables will not work |
| 1 | Smartphone (iOS or Android) | Running the Zig Sim app (free tier is sufficient) |

# 4. Hardware Design & Wiring

All pin assignments below are already reflected in the CONFIG section and pin definitions of gyro_main.py. Wiring the hardware exactly as specified avoids any need to edit pin numbers in the firmware.

## 4.1 TB6612FNG Motor Driver → ESP32

| TB6612FNG Pin | ESP32 Pin | Purpose |
| --- | --- | --- |
| PWMA | GPIO 33 | Left motor speed (PWM) |
| AIN1 | GPIO 26 | Left motor direction, bit 1 |
| AIN2 | GPIO 25 | Left motor direction, bit 2 |
| PWMB | GPIO 13 | Right motor speed (PWM) |
| BIN1 | GPIO 14 | Right motor direction, bit 1 |
| BIN2 | GPIO 12 | Right motor direction, bit 2 |
| STBY | GPIO 27 | Standby / enable — must be HIGH to drive motors |
| VM | Battery + | Motor power supply |
| VCC | ESP32 3V3 | Logic power |
| GND | ESP32 GND + Battery − | Common ground (critical) |
| AO1 / AO2 | — | Left motor terminals |
| BO1 / BO2 | — | Right motor terminals |

## 4.2 HC-SR04 Ultrasonic Sensor → ESP32

| HC-SR04 Pin | ESP32 Pin | Purpose |
| --- | --- | --- |
| VCC | 5V (VIN) | Power |
| TRIG | GPIO 32 | Trigger pulse (output) |
| ECHO | GPIO 35 | Echo pulse (input) |
| GND | Common Ground | Ground |

⚠ The HC-SR04 ECHO pin outputs 5 V, but ESP32 GPIOs are rated for 3.3 V. A voltage divider (1 kΩ in series, 2 kΩ to GND, midpoint to GPIO 35) is required to avoid pin damage. GPIO 35 is input-only, which is acceptable because ECHO is only ever read.

## 4.3 General Wiring Rules

- Common ground is mandatory: ESP32 GND, motor-driver GND, ultrasonic GND, and battery negative must all be tied together.

- Battery power feeds VM on the TB6612FNG (motor power) only — never the ESP32's 3V3/5V pins directly.

- Power the ESP32 itself via USB (for testing) or a separate regulated 5 V supply / VIN once untethered.

- Double-check battery polarity before connecting anything.

# 5. Firmware Installation & Toolchain Setup

The ESP32 ships with no interpreter on board; MicroPython firmware must be flashed before any Python code will run. This section summarises the toolchain steps from the setup guide.

## 5.1 Install Thonny

Thonny (thonny.org) is used both as the code editor and as the flashing tool. Install it for the host OS, run it once to confirm it launches, then proceed to driver installation.

## 5.2 Install the USB-to-Serial Driver

Identify the USB chip on the ESP32 board (usually printed on the chip itself):

- CP2102 / CP2104 (Silicon Labs) — install the CP210x VCP driver from silabs.com.

- CH340 / CH340C (WCH) — install the CH340 driver from wch.cn or a well-known mirror.

After installing, restart the computer — skipping this step is the most common cause of the board not being detected. Then verify the new port appears:

- Windows: Device Manager → Ports (COM & LPT) → a new COM port appears when the board is plugged in.

- macOS: ls /dev/cu.* before/after plugging in — a new /dev/cu.SLAB_USBtoUART or /dev/cu.wchusbserial… should appear.

- Linux: ls /dev/ttyUSB* before/after plugging in; may also require adding the user to the dialout group.

## 5.3 Flash MicroPython Firmware

With the ESP32 connected via a data-capable USB cable:

- In Thonny: Run → Configure interpreter…

- Set interpreter to MicroPython (ESP32); set the port (or auto-detect).

- Click Install or update MicroPython, select the target port, choose the generic Espressif ESP32 build (or the correct S3/C3 variant if applicable), and click Install.

- Flashing erases the chip and writes new firmware; it takes 1–3 minutes — do not unplug the board.

- After flashing, click Stop/Restart in Thonny; a >>> MicroPython prompt in the Shell confirms success.

⚠ If flashing fails to connect, hold the board's BOOT button as flashing starts and release once it begins. Try a different data-capable cable and a different (non-hub) USB port.

## 5.4 Upload and Configure the Firmware

- Open gyro_main.py in Thonny with the ESP32 connected.

- Edit the CONFIG section: set WIFI_SSID to include the assigned team/bot number, set a WIFI_PASSWORD of at least 8 characters, and set WIFI_CHANNEL to 1, 6, or 11 as instructed.

- File → Save As… → MicroPython device → save with the exact filename main.py, so the ESP32 auto-runs it on power-up.

- Press Run (F5) to test immediately without a reboot; the Shell should show the AP becoming active, its SSID/IP/channel, and confirmation that the UDP listener is up on port 5005.

# 6. Wireless Communication Protocol

The ESP32 does not join an existing network — it hosts its own Wi-Fi access point using network.WLAN(network.AP_IF), configured with WPA2 (authmode 3), the SSID/password/channel from CONFIG, and an optional transmit-power setting (WIFI_TX_POWER_DBM) applied on a best-effort basis via a try/except, since not all firmware builds support it.

The phone joins that access point and runs the Zig Sim app, which streams gyroscope data as OSC messages over UDP to 192.168.4.1:5005 (the SoftAP's fixed address and the port defined by UDP_PORT). The OSC address must end in /quaternion so the firmware's parser recognises it.

## 6.1 UDP Socket Behaviour

setup_udp_socket() binds a UDP socket to all interfaces on UDP_PORT and sets a zero timeout, i.e. non-blocking reads: recvfrom() either returns a packet immediately or raises OSError, which the main loop catches to move on without stalling the control cycle. Each loop iteration drains up to 8 queued packets so a backlog cannot build up and introduce latency.

## 6.2 OSC Packet Parsing

parse_osc_quaternion() supports both a single, un-bundled OSC message and a #bundle-wrapped message (Zig Sim, and OSC senders generally, may send either form):

- Bundled packets: the function skips the 16-byte bundle header, then walks each embedded element by its 4-byte big-endian size prefix, checking whether the element's OSC address ends in /quaternion; the first match is parsed and the loop stops.

- Un-bundled packets: the address string is read directly from offset 0.

- Once the matching address is found, the OSC type-tag string (e.g. ",ffff") is read, and one 4-byte big-endian value is decoded per tag — float (f) or int (i); any other tag causes the packet to be rejected.

- OSC strings are null-terminated and padded to a 4-byte boundary per the OSC spec; read_osc_string() locates the terminator and rounds the next offset up to the nearest multiple of 4.

- The function requires at least 4 decoded arguments (x, y, z, w) and rejects the packet if any value is NaN or outside ±1.0×10⁶, guarding against corrupt or malformed data reaching the motor-control logic.

- All parsing is wrapped in a try/except for ValueError, IndexError, and struct.error, so any malformed packet simply returns None rather than crashing the control loop.

⚠ Only the x and y components of the quaternion are used for control (mapped to steering and throttle respectively); z and w are decoded but not used, matching the phone's tilt-based control scheme rather than a full 3-D attitude control.

## 6.3 Phone-Side Configuration (Zig Sim Setup)

The phone is the OSC sender in this system, so it must be configured to match the ESP32's listener exactly — same network, same destination IP, same port, and an OSC address the firmware's parser will recognise. Configure it in this order:

- Install Zig Sim from the App Store (iOS) or Google Play (Android). The free tier is sufficient — it supports sending gyroscope/quaternion data via OSC.

- Join the bot's Wi-Fi network from the phone's Wi-Fi settings, using the SSID set in WIFI_SSID and the password set in WIFI_PASSWORD. The phone should receive an address in the 192.168.4.x range once connected.

- Open Zig Sim's connection/network settings and set the protocol to OSC (Open Sound Control) over UDP.

- Set the destination/target IP address to 192.168.4.1 — this is the ESP32 SoftAP's fixed address, printed in the Thonny Shell when the firmware boots.

- Set the destination port to 5005, matching UDP_PORT in the firmware CONFIG. A mismatch here is the most common reason the bot receives no data even though the phone is connected.

- Enable the Quaternion (or Rotation/Attitude) sensor in Zig Sim's sensor list, and confirm its outgoing OSC address ends in /quaternion — this exact suffix is what parse_osc_quaternion() checks for.

- Start streaming (usually a Start/Play toggle in the app). Data should now be flowing to the ESP32 at the 25 Hz rate the firmware polls at.

| Zig Sim Setting |  | Required Value | Must Match |
| --- | --- | --- | --- |
| Wi-Fi network |  | WIFI_SSID / WIFI_PASSWORD from CONFIG | AP credentials set in gyro_main.py |
| Protocol |  | OSC over UDP | Socket type opened by setup_udp_socket() |
| Destination IP |  | 192.168.4.1 | ESP32 SoftAP address (fixed for MicroPython AP mode) |
| Destination port |  | 5005 | UDP_PORT in CONFIG |
| Sensor / OSC address |  | Quaternion, ending in /quaternion | Suffix check in parse_osc_quaternion() |
| ⚠ If Zig Sim shows a connected/streaming state but the bot never moves, re-check the destination IP and port first — a typo there is the single most common configuration failure, ahead of any wiring or firmware issue. |  |  |  |

# 7. Control Algorithm

The firmware converts the phone's tilt into differential wheel speeds through three stages: dead-zone suppression, gain scaling with a speed cap, and left/right mixing.

## 7.1 Dead Zone

apply_dead_zone() ignores small values within ±DEAD_ZONE (default 0.05) — this prevents the bot from creeping when the phone is nominally level — and subtracts the dead-zone width from any value outside that band so the response is continuous at the boundary rather than jumping.

## 7.2 Gain and Speed Cap

component_to_control() multiplies the dead-zone-adjusted value by GAIN (default 3.0), then clamps the result to ±MAX_SPEED_PCT/100 (default ±0.70), so the bot never exceeds 70% of full motor speed regardless of how far the phone is tilted.

## 7.3 Differential Mixing

The y component becomes throttle and the x component becomes steering. The two are combined for classic differential-drive (tank) mixing:

left_speed = clamp(throttle + steering, -1.0, 1.0) right_speed = clamp(throttle - steering, -1.0, 1.0)

Each wheel speed is then handed to set_one_motor(), which sets the TB6612FNG's two direction pins for forward/reverse/brake and writes a proportional PWM duty cycle (0–65535) scaled from the magnitude of the requested speed.

# 8. Safety Systems

## 8.1 Obstacle Avoidance

read_distance_cm() triggers the HC-SR04 with a 10 µs pulse on TRIG, then times how long ECHO stays high, converting the pulse width to centimetres by dividing by 58 (the standard HC-SR04 constant). wait_for_echo_level() enforces a 12 ms timeout on both the rising and falling edge waits, so a disconnected or faulty sensor cannot hang the control loop — it simply returns None for that reading.

The main loop treats both a failed reading (None) and a distance below OBSTACLE_DISTANCE_CM (default 15 cm) as an obstacle condition, which forces the motors to stop. Treating sensor failure as an obstacle is a fail-safe choice: a bad ECHO connection stops the robot rather than letting it drive blind.

## 8.2 Command-Loss Watchdog

The firmware timestamps every successfully parsed quaternion. If more than COMMAND_TIMEOUT_MS (default 600 ms) elapses without a new valid command, command_timed_out becomes true and the motors are stopped — this covers the phone app closing, the Wi-Fi link dropping, or the bot moving out of range.

## 8.3 Startup and Shutdown Safety

- STBY is held HIGH only once the driver pins are initialised, and the whole control loop runs inside a try/finally block.

- On any unhandled exception or program exit, the finally clause calls stop_motors() and drives STBY LOW, cutting motor drive power regardless of how the program terminated.

# 9. Tunable Parameters

All of the following are set in the CONFIG section at the top of gyro_main.py and can be adjusted without touching the control logic itself.

| Constant | Default | Effect | If the bot feels… |
| --- | --- | --- | --- |
| DEAD_ZONE | 0.05 | Ignores small tilts near level | Creeping when flat → increase slightly |
| GAIN | 3.0 | Multiplies tilt into motor speed | Too sensitive → lower; too sluggish → raise |
| MAX_SPEED_PCT | 70 | Caps top speed (0–100) | Too fast for beginners → lower |
| OBSTACLE_DISTANCE_CM | 15.0 | Stopping distance from obstacles | Stopping too early/late → adjust |
| COMMAND_TIMEOUT_MS | 600 | Time before stop on signal loss | Keeps moving after app closes → lower |
| LOOP_PERIOD_MS | 40 | Control loop cadence (25 Hz) | N/A — affects overall responsiveness |
| WIFI_CHANNEL | 1 | Wi-Fi channel (must be 1, 6, or 11) | Set per team-lead assignment |

# 10. Commissioning & Test Procedure

⚠ Before the first powered test, prop the bot up so the wheels are off the ground, or hold it in your hands. A wiring or code error can cause unexpected full-speed movement.

- Power the ESP32 (USB is sufficient) and confirm in the Thonny Shell that the access point is active and the UDP listener is running on port 5005.

- Connect the phone to the bot's Wi-Fi network and configure Zig Sim: protocol OSC/UDP, destination IP 192.168.4.1, destination port 5005, Quaternion sensor enabled with an address ending in /quaternion.

- Temporarily uncomment the diagnostic print in the main loop, tilt the phone, and confirm changing (x, y, z, w) values appear in the Shell; re-comment it afterward since printing every cycle slows the loop.

- With wheels off the ground, tilt the phone forward/back and side/side and confirm the wheels respond correctly, and that they stop within ~600 ms of closing the app or disconnecting.

- Wave a hand within about 15 cm of the ultrasonic sensor and confirm the motors stop immediately, even while still tilting the phone.

- Only once all of the above behave correctly, set the bot on the ground and test at low speed.

# 11. Troubleshooting Guide

| Symptom | Likely Cause |
| --- | --- |
| ESP32 doesn't show up in Thonny at all | USB driver not installed, or a charge-only cable |
| Firmware install fails / times out | Hold BOOT button during flash; try another USB port/cable |
| Code uploads but nothing happens on boot | File wasn't saved as exactly main.py on the device |
| Phone can't find the bot's Wi-Fi | Wrong WIFI_CHANNEL (must be 1/6/11) or password under 8 characters |
| Zig Sim connects but bot doesn't move | Wrong destination IP/port in Zig Sim, or OSC address doesn't end in /quaternion |
| Motors twitch or run backward | Swap that motor's two output wires (AO1/AO2 or BO1/BO2) |
| Bot stops randomly even with a clear path | Check ECHO wiring/voltage divider — a bad connection reads as "no echo," treated as an obstacle |
| Bot won't stop when signal is lost | Check COMMAND_TIMEOUT_MS and confirm the phone app is actually closing the OSC stream |

# 12. Quick Reference — Full Setup Order

- 1. Install Thonny

- 2. Install CP210x or CH340 USB driver, restart the computer

- 3. Flash MicroPython firmware via Thonny

- 4. Wire TB6612FNG + HC-SR04 to the ESP32 exactly per the pin tables

- 5. Edit WIFI_SSID / WIFI_PASSWORD in the code

- 6. Save the file to the ESP32 as main.py

- 7. Install Zig Sim, connect the phone to the bot's Wi-Fi, point OSC at 192.168.4.1:5005

- 8. Test with wheels off the ground first

- 9. Tune GAIN / MAX_SPEED_PCT / DEAD_ZONE to taste

# 13. Recommendations for Future Work

- Add a low-battery cutoff by monitoring VM voltage through an ADC-safe divider, so the bot fails safe instead of behaving erratically as the battery sags.

- Extend the obstacle sensor to a second HC-SR04 or an IR array to cover side/rear collisions, since the current single forward-facing sensor only protects the front arc.

- Encode a bot ID or team number into the OSC/UDP handshake to make the swarm behavior deterministic if multiple bots' networks overlap on the same channel.

- Log tuning parameter changes (GAIN, MAX_SPEED_PCT, etc.) per bot to make it easier to reproduce a specific unit's tuned feel.

# Appendix A: Full Firmware Source — gyro_main.py

Reproduced in full below for reference. Pin numbers and CONFIG values reflect the defaults; WIFI_SSID and WIFI_PASSWORD must be edited per-bot before upload (see Section 5.4).

import network import socket import struct import time import machine # ============================== CONFIG ============================== WIFI_SSID = "SWARM_BOT_03" # Change the number of your bot with the assigned team number. WIFI_PASSWORD = "password" # Change your password within the "s. WIFI_CHANNEL = 1 # Must be 1, 6, or 11. Change according to the given instructions. WIFI_TX_POWER_DBM = 8 UDP_PORT = 5005 DEAD_ZONE = 0.05 GAIN = 3.0 MAX_SPEED_PCT = 70 COMMAND_TIMEOUT_MS = 600 OBSTACLE_DISTANCE_CM = 15.0 LOOP_PERIOD_MS = 40 ULTRASONIC_EDGE_TIMEOUT_US = 12000 ULTRASONIC_PULSE_TIMEOUT_US = 12000 # ---------- TB6612FNG motor-driver pins ---------- PWMA = machine.PWM(machine.Pin(33), freq=1000, duty_u16=0) AIN1 = machine.Pin(26, machine.Pin.OUT) AIN2 = machine.Pin(25, machine.Pin.OUT) PWMB = machine.PWM(machine.Pin(13), freq=1000, duty_u16=0) BIN1 = machine.Pin(14, machine.Pin.OUT) BIN2 = machine.Pin(12, machine.Pin.OUT) STBY = machine.Pin(27, machine.Pin.OUT) STBY.value(1) # ---------- HC-SR04 ultrasonic sensor pins ---------- TRIG = machine.Pin(32, machine.Pin.OUT) ECHO = machine.Pin(35, machine.Pin.IN) def clamp(value, low, high): if value < low: return low if value > high: return high return value def clamp_pwm(duty): return int(clamp(int(duty), 0, 65535)) def set_one_motor(pwm, in1, in2, speed): speed = clamp(speed, -1.0, 1.0) if speed > 0: in1.value(1) in2.value(0) elif speed < 0: in1.value(0) in2.value(1) else: in1.value(0) in2.value(0) duty = clamp_pwm(abs(speed) * 65535) pwm.duty_u16(duty) def set_motors(left_speed, right_speed): set_one_motor(PWMA, AIN1, AIN2, left_speed) set_one_motor(PWMB, BIN1, BIN2, right_speed) def stop_motors(): set_motors(0.0, 0.0) def apply_dead_zone(value): if -DEAD_ZONE < value < DEAD_ZONE: return 0.0 if value > 0: return value - DEAD_ZONE return value + DEAD_ZONE def component_to_control(component): value = apply_dead_zone(component) * GAIN speed_limit = clamp(MAX_SPEED_PCT, 0, 100) / 100.0 return clamp(value, -speed_limit, speed_limit) def read_osc_string(packet, offset): end = packet.find(b"\x00", offset) if end < 0: raise ValueError("OSC string has no null terminator") text = packet[offset:end] next_offset = (end + 4) & ~3 if next_offset > len(packet): raise ValueError("OSC string padding exceeds packet") return text, next_offset def parse_osc_quaternion(packet): try: offset = 0 if packet.startswith(b"#bundle"): offset = 16 while offset + 4 <= len(packet): msg_size = struct.unpack(">i", packet[offset:offset+4])[0] offset += 4 msg_end = offset + msg_size address, next_offset = read_osc_string(packet, offset) if address.endswith(b"/quaternion"): type_tags, next_offset = read_osc_string(packet, next_offset) offset = next_offset break else: offset = msg_end else: return None else: address, offset = read_osc_string(packet, 0) if not address.endswith(b"/quaternion"): return None type_tags, offset = read_osc_string(packet, offset) if not type_tags.startswith(b","): return None arguments = [] for tag in type_tags[1:]: if offset + 4 > len(packet): return None if tag == ord("f"): value = struct.unpack(">f", packet[offset:offset + 4])[0] elif tag == ord("i"): value = struct.unpack(">i", packet[offset:offset + 4])[0] else: return None arguments.append(value) offset += 4 if len(arguments) < 4: return None x, y, z, w = arguments[0], arguments[1], arguments[2], arguments[3] for value in (x, y, z, w): if value != value or value > 1.0e6 or value < -1.0e6: return None return x, y, z, w except (ValueError, IndexError, struct.error): return None def wait_for_echo_level(level, timeout_us): start = time.ticks_us() while ECHO.value() != level: if time.ticks_diff(time.ticks_us(), start) >= timeout_us: return False return True def read_distance_cm(): TRIG.value(0) time.sleep_us(2) TRIG.value(1) time.sleep_us(10) TRIG.value(0) if not wait_for_echo_level(1, ULTRASONIC_EDGE_TIMEOUT_US): return None pulse_start = time.ticks_us() if not wait_for_echo_level(0, ULTRASONIC_PULSE_TIMEOUT_US): return None pulse_us = time.ticks_diff(time.ticks_us(), pulse_start) return pulse_us / 58.0 def setup_access_point(): if WIFI_CHANNEL not in (1, 6, 11): raise ValueError("WIFI_CHANNEL must be 1, 6, or 11") ap = network.WLAN(network.AP_IF) ap.active(True) ap.config(essid=WIFI_SSID, channel=WIFI_CHANNEL, authmode=3, password=WIFI_PASSWORD) if WIFI_TX_POWER_DBM is not None: try: ap.config(txpower=WIFI_TX_POWER_DBM) print("AP TX power set to", WIFI_TX_POWER_DBM, "dBm") except Exception as error: print("AP TX-power configuration unsupported; using firmware default:", error) print("AP active:", ap.active()) print("SSID:", WIFI_SSID, "IP:", ap.ifconfig()[0], "channel:", WIFI_CHANNEL) return ap def setup_udp_socket(): udp_socket = socket.socket(socket.AF_INET, socket.SOCK_DGRAM) udp_socket.bind(("0.0.0.0", UDP_PORT)) udp_socket.settimeout(0) print("Listening for OSC UDP on port", UDP_PORT) return udp_socket def main(): setup_access_point() udp_socket = setup_udp_socket() quaternion_x = 0.0 quaternion_y = 0.0 last_valid_command_ms = time.ticks_ms() while True: loop_started_ms = time.ticks_ms() for _ in range(8): try: packet, _sender = udp_socket.recvfrom(512) except OSError: break quaternion = parse_osc_quaternion(packet) #Before connecting to the ESP32, first uncomment the below print statement and verify that your getting an output. #print("Parser output:", quaternion) if quaternion is not None: quaternion_x = quaternion[0] quaternion_y = quaternion[1] last_valid_command_ms = time.ticks_ms() distance_cm = read_distance_cm() obstacle_present = ( distance_cm is None or distance_cm < OBSTACLE_DISTANCE_CM ) command_age_ms = time.ticks_diff( time.ticks_ms(), last_valid_command_ms ) command_timed_out = command_age_ms > COMMAND_TIMEOUT_MS if obstacle_present or command_timed_out: stop_motors() else: throttle = component_to_control(quaternion_y) steering = component_to_control(quaternion_x) left_speed = clamp(throttle + steering, -1.0, 1.0) right_speed = clamp(throttle - steering, -1.0, 1.0) set_motors(left_speed, right_speed) elapsed_ms = time.ticks_diff(time.ticks_ms(), loop_started_ms) remaining_ms = LOOP_PERIOD_MS - elapsed_ms if remaining_ms > 0: time.sleep_ms(remaining_ms) try: main() finally: stop_motors() STBY.value(0)